Water Wave Mechanics ('08 Spring)

Mid-Term Exam

1 (20) The equations for an average viscous flow through a porous medium are defined by

∂p∂x=Ku

∂p∂y=Kv

where p is the hydrostatic pressure and K is a constant. Demonstrate that such a flow is irrotational. On the other hand, the equations for an average fully turbulent flow through rocks are

∂p∂x=Ku2

∂p∂y=Kv2

Demonstrate that such a flow is in general rotational.

2 (30) Often the seabed is covered with ripples. The total still-water depth, h'(x,y), can be described as h'=h(x,y)−δ(x,y), whereh(x,y) is a slowly varying depth and δ(x,y) represents rapid undulations of the depth about h(x,y) as shown below (Kirby, 1986, JFM, Vol. 162). We assume

O∇hhkh≈O(kδ)<<1, ∇h=∂∂xi+∂∂yj

(a) Find the exact bottom boundary condition.

(b) Find the bottom boundary condition to O(kδ) by expanding the bottom boundary condition about z=−h.

3 (25) A small amplitude wave of period T = 6.28 s, has traveled over a gently sloping bottom from deep water to a depth of 9 m, where it is observed to have a wave height H = 0.9 m.

- What is the wave length and the phase velocity of the wave in 9 m of water?

(b) What is the deep water wave height, H0? Assume the waves to be of normal incidence.

4 (25) Waves generated by a storm range in periods (T) from 5 to 20 seconds. Assuming deep-water conditions for the entire travel distance, apply the linear wave theory to construct a rough graph of travel time versus T for a beach 1,000 km from the storm area. How much later do the 5-s waves arrive at the beach than the 20-s waves?